SMARTCUBE ROBOT — MASTER PROJECT CONTEXT

Revised 23 September 2026 — updates from reference-project teardown and prototyping decisions are marked in shaded UPDATE blocks.

I am developing an advanced Raspberry Pi-based robotics project called SmartCube Robot.

Act as my long-term engineering partner throughout the entire project. Help me research, design, engineer, program, test, document, troubleshoot, and ultimately build the complete system.

Do not treat this as only a software project. It is a complete multidisciplinary engineering project involving:

- Robotics

- Mechanical engineering

- Electronics

- Embedded systems

- Raspberry Pi

- Microcontrollers

- Stepper motor control

- Computer vision

- Image processing

- Algorithms

- Optimization

- UI/UX

- 3D printing/CAD

- System integration

- Testing

- Technical documentation

The goal is to eventually have a polished, working physical prototype.

# 1. CORE PROJECT IDEA

SmartCube Robot is an intelligent desktop machine that can recognize twisty puzzles using cameras and help a user solve them.

Its most advanced function will be the ability to automatically scan and physically solve a standard 3×3 Rubik's Cube.

The overall system should have two main modes:

## ASSIST MODE

The user presents a puzzle to the cameras.

The system identifies or scans the puzzle and provides step-by-step instructions on the touchscreen.

Eventually this mode may support:

- 2×2

- 3×3

- 4×4

- Pyraminx

- Megaminx

- Skewb

- potentially other puzzles

The robot does NOT need to physically solve all of these.

The purpose of Assist Mode is recognition, solving calculation, teaching, and visual guidance.

## ROBOT MODE

Robot Mode is specifically designed for the standard 3×3 Rubik's Cube.

The user inserts a scrambled 3×3 cube into the machine.

The machine should then:

- Detect the cube.

- Scan its colors.

- Reconstruct all 54 facelets.

- Validate that the detected cube state is physically possible.

- Calculate a solution.

- Optimize that solution for the physical robot.

- Control six motorized face mechanisms.

- Physically solve the cube.

- Monitor execution.

- Display progress and statistics on the touchscreen.

The long-term user experience should be approximately:

INSERT CUBE
→ SCAN
→ VERIFY
→ SOLVE
→ OPTIMIZE
→ EXECUTE
→ VERIFY
→ FINISHED

# 2. EXISTING PROJECT / INSPIRATION

An important reference project is:

https://github.com/t33devv/rubik

The original idea was inspired by a video of this project.

Use this repository as an engineering reference where useful, but the objective is NOT simply to clone it.

Study its:

- mechanical approach

- V1 and V2 designs

- 3D-printable components

- electronics

- motor configuration

- Raspberry Pi software

- Arduino software

- communication protocol

- cube representation

- solving implementation

- GUI

- strengths

- weaknesses

We should reuse good ideas where appropriate while designing a substantially more capable SmartCube system.

When making important design decisions, compare our proposed solution against this reference implementation where relevant.

UPDATE — 23 September 2026

Confirmed architecture: the reference project's software runs on a Raspberry Pi (Flask server + kociemba solver), with a separate Arduino handling motor movement over USB serial — it is not Arduino-only. However, no serial-listening Arduino firmware was found anywhere in the repository, so the Pi-to-motor communication link cannot be reused directly; it will need to be built from scratch (see Section 4 update below).

Full teardown available: a complete, source-by-source teardown of this reference project — repository structure, BOM, firmware analysis, timing calculations, electronics review of the schematic, mechanical parts review, and a reuse-vs-redesign table — is recorded separately in SmartCube_Reference_Teardown.md. Treat that file as the detailed appendix to this section; it should be kept alongside this document in the project sources.

# 3. MAIN COMPUTER

The preferred main computer is:

Raspberry Pi 5

Preferably:

Raspberry Pi 5 — 8 GB

The Raspberry Pi should be responsible for high-level functions including:

- computer vision

- cameras

- cube recognition

- cube reconstruction

- solving algorithms

- motion planning

- touchscreen GUI

- statistics

- configuration

- diagnostics

- communication with the motor controller

The architecture should separate high-level computing from real-time motor control.

UPDATE — 23 September 2026

DECIDED (for prototyping): build and test on the Raspberry Pi 4 already on hand, rather than purchasing a Pi 5 up front. Revisit the Pi 5 only if vision performance testing (M7–M9) shows the Pi 4 is insufficient.

Known constraint: the Pi 4 has one 2-lane CSI camera port and one 2-lane DSI display port, versus two 4-lane MIPI ports on the Pi 5 (each usable for a camera or a display). This limits how many ribbon-cable cameras the Pi 4 can host directly; additional cameras for the multi-camera system (Section 7) would need to be USB, sharing the Pi 4's USB bandwidth. Camera count and placement remain TO BE TESTED per Section 7.

Software note: avoid GPIO libraries or approaches that are Pi 5–incompatible (e.g. code that only works with the legacy RPi.GPIO backend). Prefer a portable library such as gpiozero for any GPIO use (e.g. LED lighting control), so the software does not need rework if the project later moves to a Pi 5.

Power/thermal note: the Pi 4 draws less power (up to ~7.5 W) and runs cooler than the Pi 5 (up to ~15 W, needs a new supply), which is favorable for a compact enclosure, but sustained multi-camera vision workloads should still be tested with active cooling in mind.

# 4. MOTOR CONTROLLER

A dedicated microcontroller should handle real-time motor operation.

Candidates include:

- Arduino

- RP2040/Pico

- ESP32 if justified

The Raspberry Pi should send high-level commands to the controller.

For example:

MOVE R 90
MOVE U -90
MOVE F 180
HOME
STOP
STATUS

The microcontroller should handle:

- step timing

- acceleration

- deceleration

- motor synchronization

- homing

- limit sensors

- error states

- emergency stop behavior

Communication will probably initially use USB serial.

UPDATE — 23 September 2026

First requirements draft (from reference teardown, Section 11): the following are PROPOSED requirements for the SmartCube motion controller, informed by weaknesses found in the reference project's firmware (no serial input, no STOP, no acceleration ramp, always-on motors, sequential-only moves).

## Behavior and safety (PROPOSED)

- No motion on power-up. The controller waits for a command.

- STOP must halt motion within a bounded time; an e-stop must also cut motor power independently of the microcontroller.

- Per-driver ENABLE under controller control, with reduced idle current.

- Watchdog: if the host stops communicating, motors stop.

- Jam handling: bounded move duration and a defined response if a move cannot complete.

## Protocol (PROPOSED)

- Commands: MOVE <face> <angle>, HOME, STOP, STATUS, plus a queue for multiple moves.

- Separate acknowledgements for received, started, and completed, with error codes.

- Sequence numbers so the host can detect lost or repeated commands.

- Higher baud rate than 9600, chosen with a stress test (COMMUNICATION TEST).

## Motion (PROPOSED)

- Acceleration and deceleration ramps per move (values from TO BE TESTED experiments).

- Per-face configuration table: pins, direction invert, steps per 90°, speed limits.

- Support for executing opposite faces (R+L, U+D, F+B) simultaneously, subject to mechanical validation.

- Microstepping decision (full vs. microstep) made from noise, torque, and timing experiments.

## Verification hooks (PROPOSED)

- Report step counts and any error state so the Pi's expected-versus-actual comparison has data to work with.

Full detail and rationale for each item is in SmartCube_Reference_Teardown.md, Section 11.

# 5. SIX-MOTOR 3×3 SOLVER

The physical 3×3 solver should use six independently controlled face mechanisms.

There should effectively be one actuator corresponding to:

- U — Up

- D — Down

- L — Left

- R — Right

- F — Front

- B — Back

We originally discussed hobby servos but decided that stepper motors are probably the better engineering choice.

The reference GitHub project also uses stepper-based mechanics.

Candidate motors include appropriately sized NEMA stepper motors, with the exact size determined through torque, speed, weight, and packaging calculations.

Possible drivers include:

- TMC2209

- or another driver if engineering analysis shows it is more suitable

The mechanism must reliably perform:

- +90°

- -90°

- 180°

rotations.

Mechanical design must account for:

- cube alignment

- motor torque

- acceleration

- grip

- backlash

- dimensional tolerances

- different 3×3 cube dimensions

- friction

- spring loading/compliance

- bearings

- couplers

- face engagement

- avoiding excessive cube compression

- avoiding jams

- allowing neighboring layers to rotate

Do not assume that simply pressing six motors against a cube will work.

The mechanics must be engineered properly.

UPDATE — 23 September 2026

Reference project findings: the reference project uses A4988 drivers in full-step mode (not microstepping — MS1–MS3 tied to ground), direct-drive NEMA17 pancake motors (no gearing, 50 steps per 90°), and a 24 V motor supply with no per-driver ENABLE control and no visible bulk capacitor on the motor supply rail. TMC2209 remains our PROPOSED upgrade over A4988, pending a direct comparison on noise, microstepping, current control, and stall-detection features (see teardown Section 12, item 3).

Mechanical design still unresolved: the reference project's gripper and motor-mount geometry was only reviewed as unscaled 3D-print renders (see teardown Section 8). How neighboring grippers avoid binding the cube while one face turns is unknown and is the single biggest open mechanical question before SmartCube's own gripper can be designed.

# 6. COMPUTER VISION

Computer vision is one of the major parts of the project.

The system should ultimately recognize the cube automatically rather than requiring the user to manually enter colors.

Primary technologies:

- Python

- OpenCV

- Raspberry Pi cameras or suitable USB/global-shutter cameras if required

The vision pipeline may include:

CAMERA
→ cube detection
→ face/sticker detection
→ perspective correction
→ sticker sampling
→ color classification
→ orientation reconstruction
→ cube state validation

Do not rely only on fixed RGB thresholds.

Investigate:

- HSV

- LAB color space

- automatic white balance issues

- center-sticker calibration

- clustering

- adaptive classification

- controlled illumination

- glare reduction

- polarizing filters if useful

- machine-learning classification if justified

Lighting should be deliberately engineered into the machine.

Controlled LED illumination is preferred so recognition does not depend heavily on room lighting.

# 7. MULTI-CAMERA SYSTEM

Investigate using multiple cameras so the user does not have to manually photograph all six sides.

Possible configurations include:

- 2 cameras

- 3 cameras

- more cameras if necessary

The objective is to determine the minimum practical number of cameras capable of reconstructing all six faces reliably.

Camera placement, field of view, occlusion, reflections, resolution, synchronization, cost, and mechanical interference must all be considered.

The final camera configuration should be selected based on experiments and engineering evidence rather than assumption.

UPDATE — 23 September 2026

Interacts with Section 3: while prototyping on the Pi 4, camera count and connection type (CSI vs. USB) are constrained by the Pi 4's single CSI port. Camera experiments should record whether the chosen configuration is CSI-only, USB-only, or mixed, since this affects portability to a Pi 5 later.

# 8. CUBE STATE

For a standard 3×3 cube, the software needs to reconstruct all:

54 facelets

using the standard face notation:

U, R, F, D, L, B

Example internal representation:

UUUUUUUUURRRRRRRRRFFFFFFFFFDDDDDDDDDLLLLLLLLLBBBBBBBBB

Before attempting a solve, validate the state.

Validation should detect problems such as:

- wrong number of colors

- duplicated centers

- impossible corner orientations

- impossible edge orientations

- impossible permutations

- scanning mistakes

The GUI should explain recognition problems rather than simply failing.

UPDATE — 23 September 2026

Verified: the reference project's 54-facelet permutation tables (in cube_controller.py) were tested against the kociemba Python library over 300 random 25-move scrambles, with 0 failures (solution lengths min 11 / avg 20.5 / max 22). These tables are safe to reuse as the basis for SmartCube's own cube/state.py, wrapped with our own validation layer per this section's requirements.

# 9. SOLVING ALGORITHM

The initial solving engine should use:

Herbert Kociemba's Two-Phase Algorithm

A suitable Python implementation may be used.

We understand that Kociemba is not necessarily guaranteed to produce the mathematically shortest possible solution.

Therefore the project should accurately describe its output as something such as:

near-optimal / short solution

rather than falsely claiming that every result is the absolute minimum solution.

The solving engine should be modular so alternative algorithms could later be tested.

# 10. MOTION OPTIMIZATION

Do NOT simply send the Kociemba solution directly to the motors.

Create a separate:

Motion Planner / Motion Optimizer

For example, Kociemba might output:

R U R' U' F2 L D2 …

The motion planner should convert this into efficient physical robot operations.

Investigate:

- move simplification

- redundant move removal

- consecutive-face combination

- motor acceleration profiles

- simultaneous operations

- parallel opposite-face moves

- mechanical collision constraints

- motor thermal limits

Opposite faces may potentially rotate simultaneously:

R + L
U + D
F + B

if the mechanical design permits this safely.

An important distinction is:

Cube solver: minimize cube moves.

Robot optimizer: minimize physical execution time.

This distinction should remain central to the project.

UPDATE — 23 September 2026

Supporting evidence: a different prior-art project (rahularya50/cube) implements parallel opposite-face moves in Arduino Mega firmware and reports full scan-solve cycles under 1.5 seconds (author's claim, not independently verified). This is a useful existence proof that simultaneous R+L / U+D / F+B moves are achievable on similar hardware.

# 11. CLOSED-LOOP VERIFICATION

A major improvement over a simple cube-solving robot should be the ability to verify that the physical robot actually performed what the software commanded.

The software maintains:

EXPECTED CUBE STATE

The cameras observe:

ACTUAL CUBE STATE

Then compare:

EXPECTED == ACTUAL?

If YES:

continue.

If NO:

pause and attempt recovery or request intervention.

This could happen:

- after every move

- after groups of moves

- only when confidence drops

depending on performance.

The goal is to create a more robust closed-loop robotic system instead of blindly assuming every motor command succeeded.

UPDATE — 23 September 2026

Reference project's failure mode: the reference project's Flask app silently falls back to assuming a solved cube if its saved state file is missing or corrupt, and never scans the physical cube to check. This is precisely the failure mode closed-loop verification is meant to prevent, and is a strong argument for prioritizing this feature rather than treating it as a later polish item.

# 12. TOUCHSCREEN

The robot should include a relatively large touchscreen, approximately:

7–10 inches

with 10 inches currently preferred if the physical design allows it.

The GUI should eventually contain screens such as:

HOME
SCAN
ASSIST MODE
ROBOT MODE
CAMERA VIEW
COLOR VERIFICATION
SOLVING
3D CUBE VIEW
STATISTICS
CALIBRATION
SETTINGS
DIAGNOSTICS

The interface should feel like a finished consumer product rather than an engineering debug screen.

# 13. SOLVING SCREEN

During an automatic solve, display information such as:

SOLVING…
Move 7 / 19
Current move: R'
Progress bar
Elapsed time: 3.8 s
Estimated remaining time

A 3D cube visualization should ideally update during the solve.

# 14. ASSISTED MANUAL SOLVING

For puzzles that are not physically supported by the robot, the touchscreen should provide visual instructions.

For example:

STEP 4 / 21
R
Turn the RIGHT face clockwise.

Eventually this could include:

- animated arrows

- 3D cube animations

- move explanations

- teaching mode

The camera may also watch the user perform moves.

If the user performs the correct move:

advance automatically.

If the user performs an incorrect move:

detect the mistake and provide correction.

# 15. EDUCATIONAL MODE

An optional advanced feature is to explain WHY moves are being performed.

Instead of only:

R U R' U'

the system might explain:

“We are pairing this corner with its matching edge before inserting them into the second layer.”

This could make SmartCube useful as a teaching system rather than only a solver.

# 16. PERFORMANCE ANALYSIS

The system should record useful statistics such as:

- scan time

- recognition confidence

- solver computation time

- number of moves

- physical solve time

- time per move

- motor errors

- verification failures

- recovery events

- total solves

- fastest solve

- average solve time

For assisted human solves, potentially record:

- solve time

- hesitation

- turns per second

- incorrect moves

- repeated mistakes

# 17. SOFTWARE ARCHITECTURE

Keep the software modular.

A possible structure is:

smartcube/
main.py
config.py
vision/
    cameras.py
    calibration.py
    cube_detection.py
    sticker_detection.py
    color_classifier.py
    reconstruction.py
cube/
    state.py
    validation.py
    notation.py
solver/
    kociemba_solver.py
motion/
    planner.py
    optimizer.py
    timing.py
hardware/
    motor_controller.py
    serial_protocol.py
    sensors.py
verification/
    state_verifier.py
    recovery.py
gui/
    main_window.py
    scan_screen.py
    solve_screen.py
    settings_screen.py
    diagnostics_screen.py
analytics/
    statistics.py

This structure is only a starting point and can be changed if a better architecture is justified.

# 18. SYSTEM ARCHITECTURE

The conceptual pipeline is:

CAMERAS
↓
COMPUTER VISION
↓
CUBE RECONSTRUCTION
↓
STATE VALIDATION
↓
KOCIEMBA SOLVER
↓
MOTION OPTIMIZER
↓
RASPBERRY PI
↓
SERIAL COMMUNICATION
↓
MICROCONTROLLER
↓
STEPPER DRIVERS
↓
6 STEPPER MOTORS
↓
PHYSICAL CUBE

with feedback:

CAMERAS
↑
PHYSICAL CUBE

and the touchscreen connected to the Raspberry Pi.

# 19. SAFETY

Design safety into the machine.

Consider:

- emergency stop

- motor current limits

- thermal protection

- jam detection

- timeout detection

- pinch points

- enclosed moving components

- startup homing

- safe shutdown

- power isolation

If a cube jams, motors should not continue applying excessive force indefinitely.

UPDATE — 23 September 2026

Reference project's safety gaps: its A4988 drivers have ENABLE left unconnected (drivers cannot be switched off in software or by an e-stop), no fuse, and no visible motor-supply bulk capacitor. Its firmware also auto-runs a hardcoded solve on power-up with no stop or abort. All of these are explicitly what SmartCube's safety design (and the motion-controller requirements in Section 4) must avoid.

# 20. MECHANICAL CONSTRUCTION

The prototype will probably use a combination of:

- 3D-printed parts

- aluminum extrusion or rigid frame

- bearings

- steel shafts

- motor couplers

- fasteners

- spring-loaded components

CAD will be required.

Possible tools include:

- Fusion 360

- FreeCAD

- SolidWorks if available

We should design parts for realistic FDM printing and minimize unnecessary supports.

When designing components, specify:

- dimensions

- tolerances

- material

- print orientation

- infill

- wall thickness

- inserts

- fasteners

where appropriate.

# 21. ELECTRONICS

Develop a complete electronics architecture.

Likely components include:

- Raspberry Pi 5

- Raspberry Pi power supply

- motor-control MCU

- six stepper drivers

- six stepper motors

- cameras

- touchscreen

- sensors

- emergency stop

- cooling

- LEDs

- motor power supply

The motors should NOT be powered directly from the Raspberry Pi.

Separate motor and logic power properly while maintaining the necessary common references.

Eventually create:

- wiring diagram

- connector map

- power budget

- fuse/protection design

- PCB or distribution-board design if justified

# 22. BILL OF MATERIALS

Maintain a continuously updated BOM.

For every component record:

- component

- manufacturer

- exact model/part number

- quantity

- purpose

- important specifications

- supplier

- approximate price

- alternatives

Avoid selecting components simply because they are cheap.

Explain engineering trade-offs.

# 23. TESTING STRATEGY

Do not wait until the complete robot exists before testing.

Develop subsystem tests.

Examples:

MOTOR TEST

Command 10,000 repeated quarter-turns and measure accumulated positioning error.

VISION TEST

Scan multiple cube brands under different lighting conditions.

COLOR TEST

Measure confusion between red/orange and white/yellow.

SOLVER TEST

Generate thousands of valid random cube states and verify solutions.

COMMUNICATION TEST

Stress-test Raspberry Pi ↔ MCU communication.

JAM TEST

Verify safe behavior when a face cannot rotate.

FULL SYSTEM TEST

Scramble → scan → solve → verify.

Keep measurable results.

# 24. DEVELOPMENT PHILOSOPHY

Build incrementally.

Do NOT attempt the entire machine at once.

Suggested milestones:

- M0 — Research and requirements

- M1 — Software cube representation

- M2 — Kociemba solver working

- M3 — One motor rotates one cube face reliably

- M4 — Six-motor mechanical prototype

- M5 — MCU motor controller

- M6 — Manual cube-state entry produces automatic physical solve

- M7 — Single-camera color recognition

- M8 — Multi-camera recognition

- M9 — Fully automatic scan

- M10 — Touchscreen GUI

- M11 — Closed-loop verification

- M12 — Motion optimization

- M13 — Final enclosure

- M14 — Testing and benchmarking

- M15 — Final documentation and demonstration

Every major milestone should be independently testable.

# 25. WHAT I EXPECT FROM CLAUDE

Act as a senior engineering mentor and project collaborator.

When helping me:

- Challenge weak design assumptions.

- Explain why a solution is appropriate.

- Give alternatives when meaningful.

- Consider cost, complexity, reliability and manufacturability.

- Prefer testable engineering decisions over guesses.

- Keep the complete system architecture in mind.

- Warn me when one decision creates problems elsewhere.

- Help me write production-quality code.

- Help design experiments before choosing components.

- Help maintain documentation as the project evolves.

Do not automatically agree with my ideas.

If there is a technically superior approach, explain it.

When we have not yet made a decision, distinguish clearly between:

- DECIDED

- PROPOSED

- TO BE TESTED

# 26. RESEARCH

When current information matters, research it rather than guessing.

Examples:

- Raspberry Pi hardware

- camera specifications

- stepper motors

- motor drivers

- touchscreen compatibility

- GitHub libraries

- existing cube robots

- component prices

- CAD references

- software libraries

Prefer:

- official documentation

- datasheets

- original repositories

- academic papers

- reputable engineering sources

Always distinguish confirmed specifications from assumptions.

# 27. DOCUMENTATION

Throughout development, help maintain professional documentation including:

- project specification

- architecture

- requirements

- design decisions

- diagrams

- BOM

- wiring

- mechanical drawings

- software documentation

- algorithms

- testing

- results

- problems encountered

- solutions

- future improvements

The eventual goal is to produce an extensive final engineering report and build manual.

UPDATE — 23 September 2026

Active project documents: Untitled document.docx (this file) is the master specification for SmartCube itself. SmartCube_Reference_Teardown.md is the companion document analyzing the reference GitHub project (t33devv/rubik). Keep both in the project sources; the teardown should be updated as new reference material (e.g. the demo video, measured 3D part dimensions) is reviewed.

# 28. FINAL PROJECT TARGET

The finished machine should ideally behave approximately like this:

The user approaches SmartCube Robot.

The touchscreen shows:

SMARTCUBE
[ ASSIST MODE ]
[ ROBOT SOLVE ]

The user selects ROBOT SOLVE.

The screen says:

INSERT 3×3 CUBE

The user inserts a scrambled cube.

The machine detects it.

LED lighting activates.

The cameras scan the cube.

The display shows:

SCANNING…
Cube detected ✓
54 stickers recognized ✓
State valid ✓

Then:

CALCULATING SOLUTION…
Solution: 19 moves
Optimized robot operations: 15
Estimated time: 5.8 seconds
[ SOLVE ]

The user presses SOLVE.

The six-motor mechanism executes the solution.

The display shows a live 3D representation and progress.

The cameras periodically verify the physical state.

Finally:

SOLVED ✓
Time: 5.6 seconds
Cube moves: 19
Robot operations: 15
Recognition confidence: 99.4%

The robot then releases the solved cube.

That is the long-term vision.

# 29. IMPORTANT SCOPE CONTROL

The project is intentionally ambitious, but it must remain buildable.

The core required achievement is:

A Raspberry Pi-based computer-vision robot capable of automatically recognizing and physically solving a 3×3 Rubik's Cube using six motorized face mechanisms.

Everything else should be treated as an extension.

Therefore prioritize:

RELIABLE 3×3 ROBOT

before:

- multiple puzzle types

- cloud services

- voice assistant

- mobile apps

- AI features

- advanced analytics

A reliable physical prototype is more important than many unfinished features.

# 30. STARTING POINT

The first reference implementation is:

https://github.com/t33devv/rubik

Begin by helping me reverse-engineer and understand this project.

Then help me create our own SmartCube architecture.

Before purchasing hardware or designing the final chassis, we should determine:

- Exactly how the reference robot works.

- Which parts of its architecture are worth reusing.

- Which parts should be redesigned.

- Required motor torque and speed.

- Suitable stepper motors and drivers.

- Camera count and placement.

- Raspberry Pi architecture.

- MCU choice.

- Mechanical cube-gripping design.

- Initial BOM.

- Development milestones.

From that point onward, treat this prompt as the master specification for the SmartCube Robot project and help me evolve it into a real working machine.

UPDATE — 23 September 2026

Status: Section 30's reverse-engineering task is substantially complete — see SmartCube_Reference_Teardown.md for the full writeup. Remaining open items from that teardown (Section 12): confirm the reference's gripper/neighbor-clearance behavior (needs the demo video or measured 3D parts), get the actual NEMA17 pancake motor datasheet for torque calculations, compare A4988 vs. TMC2209 directly, and define the camera count/lighting experiments referenced in Section 7 above.